Abstract

Road accidents involving two-wheeler riders often result in delayed emergency response, significantly increasing fatality rates during the critical “golden hour.” To address this challenge, we propose a Smart Helmet for Accident Detection and Emergency Response, an IoT-enabled system designed to automatically detect crashes and instantly alert emergency contacts with vital information.

The system integrates an ESP32 microcontroller with multiple sensors and communication modules to ensure real-time monitoring and rapid response. An ADXL335 accelerometer continuously tracks motion and detects sudden impact beyond a predefined threshold, while a pulse sensor monitors the rider’s heart rate. Upon detecting a potential accident, a 5-second buffer mechanism is activated using a buzzer and override button to minimize false alarms. If not canceled, the system retrieves precise location data using a NEO-6M GPS module and sends an SOS alert via using twilio and blynk, including location coordinates and live health data. Additionally, the system updates data to a cloud platform using IoT integration for remote monitoring.

Smart helmet for accident detection.pptx None

To enhance medical response, the system can be upgraded with advanced sensors like MAX30102 for SpO₂ monitoring, enabling better triage decisions by paramedics. Future enhancements include proactive safety features such as blind-spot detection and haptic feedback to prevent accidents before they occur.

This solution aims to reduce emergency response time, improve survival rates, and bring intelligent safety integration into everyday transportation.